Direction Cobb - June 2001

Randy Drinkard, Cobb County Extension Service

Drip Irrigation Saves Time, Money and Plants

Do you remember how hot and dry it was in Cobb County last summer? And the summer before? And the summer before that? Well, this year’s scortching heat is just around the corner, so odds are very good that your landscape and foundation plants, flowering shrubs, fruit trees and other plants will need supplemental watering to make it through the summer of 2001.

Before you resign yourself to spending a lot of time and energy watering with a garden hose, you might want to consider installing a drip irrigation system. Drip irrigation not only makes it easier to water your plants, but this method utilizes water more efficiently, too. The biggest advantage of using drip irrigation is that it directs a precise amount of water to the place where it is needed most - the plant’s root system - while at the same time it reduces water loss through evaporation and runoff. By contrast, if you hand water plants with a garden hose, you can actually hurt them. For instance, if you water your plants often and for just a few minutes, you are encouraging the roots to grow only into the top several inches of soil, which in fact makes your plants less drought-tolerant. On the otherhand, if you overwater, you may encourage the development of  root rot diseases that could kill you plants.

The primary reason that gardeners are hesitant to install a drip irrigation system is cost - they simply think it will cost too much.  Although a drip system might look expensive at first, if considered over the long haul, installing and utilizing drip irrigation will cost much less than you think. If you install it yourself, a drip system that will irrigate all the shrubs and small trees around an average yard would typically cost about $200 or less. For that amount of money and not a lot of work, you can simply turn a valve on that 100-degree day this summer and walk back inside where it’s cool and relax while your plants are being effeciently watered. Better yet, you can invest about $100 more and make your system fully automatic. Another huge advantage of using a drip irrigation system is that it uses up to 50% less water than other sprinkler systems. There are some limitations to using a drip irrigation system, however. Drip systems work well for watering shrubs, trees, annuals and perennial flower beds and even home vegetable gardens, but if you intend to water your lawn, a sprinkler system is still your best choice.

A simple drip system can be installed in a single afternoon by the typical homeowner. Most components of the system are plastic and require few tools. The most widely-used type of drip irrigation uses individual drippers, or emitters, that are attached to half-inch-diameter black tubing. The emitters are precalibrated for a particular output - usually one-half, one or two gallons per hour. The other components needed are a valve, filter(with a 200-mesh nylon screen) pressure regulator and fittings to connect everything together.

You can purchase drip irrigation kits from most garden or home imprrovement centers , or yu can buy individual components from most irrigation retailers. Once the controls are installed, just lay out the tubing around the plants that you need to water and put the emitters where they are needed. A simple guide is to place a one-gallon emitter by each plant that is 2 to 4 feet high, two one-gallon emitters by each plant that is 4 to 6 feet high and four one-gallon emitttters (or two two-gallon emitters) by each plant that is eight feet high.

cond.

Emitters are easy to install - simply  punch a hole in the tubing with a special punch and snap the emitter into place. In beds, with mulch, hide the tubing under the mulch material. In cases where the the tubing must cross  turf areas to get from one bed to another, simply bury the tubing a few inches deep in the soil under the grass.

Now comes the fun(and easy) part;all you have to do is turn the system on for three or for hours twice a week during hot, dry weather and sit back and relax knowing that all your landscape plants are well-watered and happy.

For more information on this subject or related subjects, like xeriscaping and drought-tolerant plants, call or visit the Cobb County Extension Service. Our telephone number is 770/528-4070 and our office is located at 678 South Cobb Drive in Marietta.

## Ornamental Plants Known to Have Above-Average Drought Tolerance

Ground Covers/Shrubs:			      Annuals/Perennials:

Abelia						Annual phlox

Aucuba					Annual periwinkle

Barberry					Asclepias

Beautyberry					Baby’ breath

Butterfly bush					Black-eyed Susan

Cotoneaster					Calendulia

Crepe myrtles					Candytuft

English ivy					Canna

Daylily						Coreopsis

Forsythia					Cornflower

Hollies						Cosmos

Junipers					Gaura

Lantana					Gazania

Ligustrum					Gaillardia

Liriope						Globe amaranth

Mondograss					Moss rose

Nandina					Sedum

Pampas grass 					Stokesia

Spirea 						Strawflower

Vibernum 					Verbena

Wax myrtle 					Yarrow